# MiniJoyC Remote (M5StickC Plus2)

# MiniJoyC Remote for ProPresenter

A compact, joystick‑driven ProPresenter remote originally designed for the M5Stack M5StickCPlus, now adapted for the **M5StickC Plus2**.

This version preserves the original M5StickCPlus UI and behavior while running on modern hardware.

Features

- NEXT / PREV slide control

- Slide number display

- Title marquee

- Status bar with Wi‑Fi and ProPresenter indicators

- Heartbeat icon

- Auto‑dismiss network info page

- Clean, minimal UI

Hardware Required

- M5StickC Plus2**

- M5Stack MiniJoyC (I²C joystick)**

- USB‑C cable

- Wi‑Fi network

- ProPresenter 7

Installation Instructions

1. Install Arduino IDE

Download from: https://www.arduino.cc/en/software

2. Install ESP32 Board Support

1. Open Arduino IDE

2. Go to **File → Preferences**

3. Add this URL:  https://espressif.github.io/arduino-esp32/package_esp32_index.json

4. Go to **Tools → Board → Boards Manager**

5. Install **ESP32 version 3.3.5**

3. Install Required Libraries

Install via **Tools → Manage Libraries**:

- **M5StickCPlus2** (1.0.1)

- **M5Unified** (0.2.11)

- **M5GFX** (0.2.18)

- **ArduinoJson** (7.4.2)

- **MultiButton** (1.3.0)

4. Select the Correct Board

**Tools → Board → ESP32 → M5Stack M5StickC Plus2**

5. Open the Project

Open: MiniJoyC_Remote/MiniJoyC_Remote.ino

6. Upload to the Device

Click **Upload**.

Editing Profiles

Profiles are defined near the top of the `.ino` file:

{ "Home PC", "MySSID", "MyPassword", "192.168.1.10", 5005 },

Each profile includes:

- Display name

- Wi-Fi SSID

- Wi-Fi password

- ProPresenter host IP

- ProPresenter port

Usage

- Joystick UP/DOWN triggers NEXT/PREV

- Button A dismisses info screens

- Status bar shows Wi-Fi and ProPresenter reachability

- Heartbeat blinks every 500ms

Notes

- ProPresenter must have the Network API enabled

- Device must be on the same network as ProPresenter

- MiniJoyC joystick uses I²C address 0x38

License

This project is provided as-is for personal and educational use.